CESSNA 140 Checklist

Specifications for Continental O-200-A

SPECS

- MTOW	1450 (lbs)

- OEW	974 (lbs)

- Payload	476 (lbs)

- Useable Fuel (minimum 3g each tank)	19 (g)

- Oil (absolute max 7qt)	4qt. (min) 5qt. (max)

- Oil Consumption	0.6 Qts/h

- Tire Pressure Main	16 to 18 (lbs/sq in)

- Tire Pressure Tail	15 to 30 (lbs/sq in)

AIRSPEED

- Pwr On Stall Dirty (+3 kts Clean)	32 (KIAS)

- Vs0	39 (KIAS)

- Vs	43 (KIAS)

- Short Field (Flaps 40°)	51 (KIAS)

- Approach (Flaps 40°)	56 (KIAS)

- Approach (Flaps Up) / Vbg (~1.5NM per1k’)	61 (KIAS)

- Vx	56 (KIAS)

- Vy	63 (KIAS)

- Normal Climb	64 to 70 (KIAS)

- En Route Climb	70 to 80 (KIAS)

- Vfe	71 (KIAS)

- Va (1100 lbs)	80 (KIAS)

- Va (1300 lbs)	87 (KIAS)

- Va (1450 lbs)	92 (KIAS)

- Vno	100 (KIAS)

- Vne	122 (KIAS)

LIMITATIONS

- Tach (rec 60 bhp)	2000 to 2600 (rpm)

  - Cruise: 2400 (Low/Cold) -2600 (Hot/High)

- Tach No Go -20°C ISA <8k’	2600 to 2750 (rpm)

- Tach Max 5m climb	2750 (rpm)

- Oil Pressure	R: <10 - G: 30 to 60 - R>100 (psi)

- Oil Temp	Y: <75° -G: 75° to 225° - R: >240° (F)

- Cylinder Head (rec. <350)	Y: <200° - G: 200° to 400° - R: >525° (F)

PERFO.

- Takeoff Roll @ ASL/ISA	663’

- Landing Roll @ ASL/ISA	302’

- Max. Crosswind	9 kts

- Fuel (78% bhp)	5.9 (g/hr)

- Range @ ASL/ISA (3.5g)	6h17m

- Range (>ASI drop): Fly High / Endurance (=Alt): Fly Low

- Service Ceiling	15 500’

Performance Chart

+0.8g Runup

>time, fuel & distance by 10% for each 8°C above ISA

Recommended Lean Mixture (from C150)

*Lean ROP - EGT: +50°F to +100°F – CHT: <350°F

| IAS | 40 | 60 | 80 | 100 | 120 | 140 |
| --- | --- | --- | --- | --- | --- | --- |
| CAS | 46 | 65 | 83 | 100 | 116 | 133 |

PREFLIGHT

COCKPIT

- Mag	OFF

- Fuel	CHECK QTY

- Rudder & Brake Linkage	SECURE

- Hydraulic Brake Reservoirs	NO LEAKS

- Control Cable Attachment	SECURE

- Seat belts (no passenger)	STOWED

- AROWJIL	SECURE

LANDING GEAR

- Attachments	SECURE

- Hydraulic lines	NO LEAK (Red Fluid)

- Brakes	CHECK

- Tire & wheel hub	CHECK

- Gear Box	NO CRACK

WING

- Strut attachment	SECURE

- Aileron attachment	SECURE

- Aileron	MOVE FREELY

- Spar check	SOME FLEX

- Fabric	NO DAMMAGE & WRINKLES

- Leading Edge	NO CONTAMINENTS

- Check fuel level	25g MAX

- Fuel Cap	SECURE

ENGINE

- Windshield	CLEAN

- Engine Oil Level	4 MIN 5 MAX

- Engine Oil Dipstick	SECURED

- Fuel drain	SAMPLE

- No Significant Oil Leaks

- Propeller & Spinner	DENT/NICK

- Cowling pins	SECURE

- Exhaust pipe	NO HOLES/CRACKS

AIRFRAME

- Skin (including belly)	DAMMAGE

- Rivet	NO GRAPHITE COLOR

TAIL

- H-Stab & Trim	SECURE & FOD FREE

- Flying wires & bolts	SECURE

- V-Stab	SECURE

- Elevator & Rudder hinges	SECURE

- Rudder & Elevator	MOVE FREELY

- Steering springs and chains	IN PLACE

- Leaf spring (check bolts)	SECURE

ENGINE START

- Time	RECORD

- Radio	OFF

- Master	ON

- Fuel tank	LOWEST TANK SELECTED

- Carb Heat	OFF

- Mags	ON

- Prime	3 to 5 TIMES (COLD ONLY)

- Brakes	SET

- Throttle	1/4’ CRACKED (800 rpm)

- Stick	FULL AFT

- Starter	PUSH

- Oil Pressure	(30s Warm / 60s Cold) CHECK

- Throttle	SET 1000 (rpm)

- Radio	ON

*Crack the throttle to ½’ & up if the engine doesn’t start. Use primer as req.

BEFORE TAXI

- Radio	CHECK

- Altimeter	SET

- Brakes	TEST

- Taxi	DON’T RIDE THE BRAKE

RUN UP

- AC	INTO THE WIND

- Run-up	1700 rpm (Hold Brake Stick Back)

  - Mag Dead	MAX Drop 150 & DIF 75

  - Carburetor heat	CHECK

- Oil Pressure/Oil Temperature	CHECK

- Primer	IN & LOCKED

- Instruments	CHECK

- Fuel	FULLEST TANK SELECTED

- Trim	SET FOR TAKEOFF

- Control	FREE & CORRECT

- Engine Temp (m75°F)	(2m Warm/4m Cold) CHECK

FLIGHT

NORMAL TAKEOFF

- Check wind and apply correction

- Smoothly apply full power

- Check oil temp (75-225°F) / Oil pressure (30-60psi)

- Neutralize controls as aircraft accelerates

- Set slight nose high climb attitude

- Climb Speed 65 kias

WHEEL TAKEOFF

- Check wind and apply correction

- Smoothly apply full power

- Ease stick forward to raise tail

- Check oil temp (75-225°F) / Oil pressure (30-60 psi)

- As tail rises smoothly reduce forward stick pressure to hold aircraft level

- Slight back pressure to fly

- Climb Speed 65 kias

SHORT-FIELD TAKEOFF

- Check wind and apply correction

- Hold full back

- Hold full brake

- Smoothly apply full power

- Check oil temp (75-225°F) / Oil pressure (30-60 psi)

- Release the brakes

- Tail stays low (about 6’ of runway surface)

- Climb Speed 55 kias

IN FLIGHT CHECK (15m)

- Carb Heat	ON

- Fuel	CHECK (ALLOW ¼ TANK DIFF. MAX)

- DG	ALIGN WITH COMPASS

NEEDED DESCENT

- Carb Heat	ON

- Throttle	~1500 RPM

3 POINT LANDING

- Carb Heat	ON

- Primer	IN & LOCKED

- Fuel	CHECK

- Approach Speed	50-60 kias

- At touch down stick full back

WHEEL LANDING

- Carb Heat	ON

- Primer	IN & LOCKED

- Fuel	CHECK

- Approach Speed	50-60 kias

- Allow mains to roll smoothly on to the ground

- After the mains are down apply forward stick pressure

- Keep tail up as plane slows

- After tail touches apply full stick full back

SECURING PLANE

- ELT	LISTEN 121.5MHz

- Radio	OFF

- Throttle	LOWEST

- Mags Live Cut	CHECK

- Throttle	1000 rpm

- Mixture	FULL LEAN

- Mag	OFF

- Master	OFF

- Time	RECORD

EMERGENCY PROCEDURES

ENGINE FAILURE

DURING TAKEOFF ROLL

- Throttle	IDLE

- Brakes	APPLY GENTLY

IMMEDIATELY AFTER TAKEOFF

- Best Glide	60 kias

- Landing Site	SELECT

- Fuel	OFF

- Magnetos	OFF

ENGINE FAILURE INFLIGHT

- Best Glide	60 kias

- Field	SELECT

- Approach	HOW

- Carburetor Heat	ON

- Fuel	CONFIRM ON / LEVEL

- Magnetos	ON

- Engine Gauges	CHECK

- Primer *	IN & LOCKED

- Magnetos	Left / Right / Both

- Throttle	FULL FORWARD

*Primer can be used with Carb Ice

IF ENGINE RESTARTS

- Carburetor Heat	AS NEEDED

- Land as soon as possible

EMERGENCY DESCENT

- Radio	MAYDAY

- Magnetos	OFF

- Forced Landing	EXECUTE

*GFA CRAB

FORCED LANDING

- Best Glide	60 kias

- Radio	MAYDAY

- Fuel	OFF

- Magnetos	OFF

- Door	OPEN

- Touchdown Speed	50 kias

ENGINE ROUGHNESS

- Carburetor Heat	ON

- Engine Gauges	CHECK

ENGINE FIRE In FLIGHT

- Fuel	OFF

- Throttle	CLOSED

- Cabin Heat	OFF

- Airspeed	>85 kias

SPINS

- Power	IDLE

- Ailerons	NEUTRAL

- Rudder	OPPOSITE ROTATION

- Elevator	RELEASE BACKPRESSURE

*Do not exceed Vne of 122 knots

LOSS OF OIL PRESSURE/ HIGH OIL TEMPERATURE

- Land as soon as practical

- Prepare for imminent engine failure